CHIMNEY RIDGE CONDOMINIUMS

REQUEST FOR PROPOSAL

EXTERIOR REPAIRS

Chimney Ridge, a condominium community in the Woodcroft Community of Durham, North Carolina, is seeking bids on the following scope of work.  The attached chart is the amount of rotten wood we have estimated.  Pricing should reflect this quantity for the scope listed.

Siding/Trim Repairs:

- Remove and replace rotten/deteriorated corner boards, rakes, fascia with Miratec to closely match existing design and footprint

- Remove and replace rotten and deteriorated brick molding and sill nosing with PVC to closely match existing design and footprint

- Remove and replace rotten 2x freeze boards with pressure treated lumber

- Remove and replace rotten siding with new T1 11 siding to closing match existing design

- Any unforeseen adjacent damages to be repaired per the following:

- Sheathing/Siding:   $______ per 4x8 sheet

- Trim and Framing:  $______ per Linear Foot

Repaint Scope:

- Chem/Power wash all siding and trim components to remove dirt, grime, and mildew

- Scrape all delaminating paint

- Prime all bare wood areas

- Replace all missing or compromised caulking

- Repaint all previously field finished surfaces with one coat of Sherwin Williams Duration Paint to closely match existing

- Repair rotten siding and trim components as outlined below

- All gutters to stay secured.  Downspouts to be detached and reset as needed.

- Excludes painting/staining rear decks and partitions

Pre-paint Chemwash:

- Chem/power wash all siding and trim components to remove dirt, grime and mildew

- Excludes manipulating porch furniture, plants, etc.  Homeowner to move contents off porch prior to completion

- Price assumes chemwash will be completed in conjunction with repaint project

- Use of chlorine involved in low-pressure washing process.  All landscaping at affected areas will be rinsed both before and after building wash with fresh water to limit any possible damage to vegetation.

Exterior Decks:

- Provide a price to prep and stain all exterior decks with Benjamin Moore 984 Stonehearth stain.